Windows 95 Help Topic

This provides the information you need to connect to the internet with Windows 95 using the windows 95 help file.  Avon Global comments are contained in bold italics like this.

Get to the help file by clicking on:-

Help

Help Topics

Index

Dial Up Networking

Connecting to the Internet

The Help File then steps through the following processes.

There are many ways you can connect to the Internet using Windows. Here are two ways you can connect.

You can sign up for The Microsoft Network online service.

You can create a Dial-Up Networking connection to an Internet access provider.

How to sign up for The Microsoft Network

The help file goes through setup to MSN.  Don’t use this for access through Avon Global.

How to connect to the Internet using Dial-Up Networking

This is the option you require to log in with Avon Global.

Connecting to the Internet using Dial-Up Networking

The steps in this procedure will guide you through the process of connecting to the Internet by using Dial-Up Networking. Just click the button to start, or click Quick Overview to see a summary of the steps.

Quick overview

Here's a quick look at the steps required for connecting to the Internet by using Dial-Up Networking:

1	Get an Internet account with an Internet access provider.

2	Set up your communications hardware.

3	Make sure Dial-Up Networking is installed.

4	Install the TCP/IP protocol and bind it to the Dial-Up Networking adapter.

5	Enter TCP/IP information.

6	Create a connection to your access provider in Dial-Up Networking.

7	Dial your Internet access provider.

Step 1  Getting an Internet account

When you call an Internet access provider to sign up for Internet account, make sure you get the following information:

User name

Any combination of letters and characters in lowercase to maximum of 8 characters.

Confirm your user name with Avon Global  before trying to connect

Password

Discuss this with Avon Global before trying to connect.

Access phone number

227177 for areas which don’t use 096 to dial Northam, such as Northam, York etc

096 227177 for areas that use 096 to access Northam such as Toodyay

574 4443 For areas which are local call to Toodyay but STD to Northam such as Gidgegannup

Other numbers will be provided as members join from outlying areas.

Avoid STD rates!  Check with Avon Global if you are not sure which number to install.

Your host name and domain name

The host name for this installation is your user name.

Domain name is avon.net.au

Domain Name Server (DNS) server address

The server address is 203.59.31.1

The access provider may also provide the following information:

IP address and subnet mask

Your IP number is allocated dynamically each time you log in.

DNS search order (if required)

DNS Server Search Order is

203.59.31.1

DNS Suffix Search Order is

avon.net.au

Gateway address

Leave as it is.  This is allocated when you log in.

Authentication procedure (whether or not they require a terminal window to log on)

Yes you need the terminal window after logging in.

Note

Use a PPP account if your provider offers it.

Yes Avon Global gives you PPP connection.

Point-to-Point protocol is a communications protocol for transmitting information over standard telephone lines. You need a PPP account so you can connect to your Internet access provider's PPP server. Windows comes with support for PPP connections.

If your organization has a direct connection to the Internet, you can skip this step.

What's next?

I have an Internet account. Go to step 2.

Step 2  Setting up your communications hardware

You need a modem (9600 bps or faster is recommended) to connect to the Internet. Or, if your organization has a direct connection to the Internet, you can use a network adapter card instead of a modem.

If you haven't set up your modem, follow these steps:

1	The help file provides a button to set up a modem.

2	Follow the instructions on your screen.

What's next?

The modem is set up. Go to step 3.

Step 3  Make sure you have Dial-Up Networking installed

1	Double-click the My Computer icon on your desktop. If you do not see a Dial-Up Networking icon, then follow steps 2 through 4.

2	Click the button in the help file to display Add/Remove Programs.

3	Click Communications, and then click Details.

4	Check Dial-Up Networking, and then click OK.

What's next?

Dial-Up networking is installed. Go to step 4.

Step 4  Install the Windows 95 TCP/IP protocol

1	Click on help button  to display Network properties.

or

My computer

Control Panel

Network

2	Click Add. (If the Windows 95 TCP/IP protocol is already installed, you can go to step 6.)

3	Click Protocol, and then click Add.

4	Click Microsoft, and then click TCP/IP.

5	Click OK.

6	Click Dial-Up Adapter, and then click Properties. Or, if your organization has a direct Internet connection, click your network adapter, and then click Properties.

7	Click the Bindings tab, and then make sure the TCP/IP check box is checked.

If prompted, restart your computer. Then, start Help and return to this procedure.

Note

When you install TCP/IP, it is bound to all of your adapters by default. If you have a network card and don't use TCP/IP with it, you should go to Properties for the network card, click the Bindings tab, and then remove TCP/IP.

What's next?

The adapter is bound to TCP/IP. Go to step 5.

Step 5  Entering TCP/IP information

1	Click button in Help topic to display Network properties.

or

My computer

Control Panel

Network

2	Click TCP/IP (If there is more than one TCP/IP entry, click TCP/IP -> Dial-Up Adapter), and then click Properties.

3	On the IP Address tab, if your provider gave you an IP address, click Specify An IP Address, and then type your IP address and subnet mask.

Leave the specify button blank

If your provider did not give you an IP address, click Obtain An IP Address Automatically.

Click Obtain An IP Address Automatically.

If you are using a direct network connection to the Internet, type your IP address and subnet mask. If your network uses a DCHP server to assign IP addresses, click Obtain An IP Address Automatically.

4	On the Bindings tab, it is recommended that you clear the File And Printer Sharing box.

On the DNS Configuration tab

Click Enable DNS

Host is your username

Domain is avon.net.au

DNS Server Search Order is

203.59.31.1

Click Add

Domain Suffix search order is

avon.net.au

Click Add

WINS Configuration tab

Disable should be clicked.

Gateway tab

should be blank

Advanced tab

should be none

What's next?

My TCP/IP information is entered. Go to step 6.

Step 6  Setting up a connection to your access provider

1	In Dial-Up Networking, double-click Make New Connection. (If it is your first Dial-Up Networking connection, you’ll be prompted automatically.)

2	Follow the instructions on your screen.

Type Avon Global for the name

Check your modem settings

General Tab speed settings should be:-

For 28800 use 38400

For 14400 use 19200

For 9600 try 19200 or 9600

Phone number

227177 for areas which don’t use 096 to dial Northam, such as Northam, York etc

096 227177 for areas that use 096 to access Northam such as Toodyay

574 4443 For areas which are local call to Toodyay but STD to Northam such as Gidgegannup

Other numbers will be provided as members join from outlying areas.

Avoid STD rates!  Check with Avon Global if you are not sure which number to install.

Country code is Australia 61

3	Using your right mouse button, click the connection icon, and then click Properties.

4	If your provider requires a terminal window to log in, click Configure. Click the Options tab, make sure Bring Up Terminal Window After Dialing is checked, and then click OK.

Yes check Bring Up Terminal Window After Dialing

5	In the Properties for the connection, click Server Type, and then make sure the type of server is set to PPP.

You can decrease the time required to connect by making sure that the following options are not checked:

Log On To Network

NetBEUI

IPX/SPX Compatible

Note

If you have a SLIP account, you need to install SLIP support. This is available on the Windows 95 CD-ROM or from the Windows 95 Resource Kit. After SLIP support is installed, go to the properties for your connection, click Server Type, and make sure SLIP is installed.

The Avon Global account is PPP

What's next?

My connection is set up. Go to step 7.

Step 7  Dialing your Internet access provider

Double-click the connection, enter your name and password, and then click Connect.

Follow the instructions provided by your Internet access provider for browsing the Internet, reading e-mail, and accessing information.

A terminal screen will then prompt you

type your username and press return

type your password and press return

type ppp and press return

Press F7

And you are on the Net.  You can now access the Internet through your Internet software.

Notes

Information transfer on the Internet is bidirectional, meaning that while you are connected, other people can gain access to your computer. For this reason, it is recommended that you disable file and printer sharing before connecting.

For information about how to change protocol settings, how to display a logon screen after dialing, how to disable sharing, or how to connect to other operating systems, click Related Topics.